Chapter 01 Test Bank: Introduction

1.	A formula is a rule that relates quantities.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.01 The Three Kinds of Formulas

Subtopic: The Three Kinds of Formulas

Topic: Introduction

2.	A definition is a formula invented for a new concept.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.01 The Three Kinds of Formulas

Subtopic: The Three Kinds of Formulas

Topic: Introduction

3.	A law summarizes a relationship that does not yet exist in nature.

FALSE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.01 The Three Kinds of Formulas

Subtopic: The Three Kinds of Formulas

Topic: Introduction

4.	A law is true because you can verify it with an experiment.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.01 The Three Kinds of Formulas

Subtopic: The Three Kinds of Formulas

Topic: Introduction

5.	A derivation is a formula produced without the use of mathematics.

FALSE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.01 The Three Kinds of Formulas

Subtopic: The Three Kinds of Formulas

Topic: Introduction

6.	The ideal approximation is sometimes called the

A.

A. first approximation.

B.

B. second approximation.

C.

C. third approximation.

D. fourth approximation.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.02 Approximations

Subtopic: Approximations

Topic: Introduction

7. Which approximation is generally the best choice because it is easy to use and does not require a computer?

A.

A. first

B. second

C. third

D. fourth

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.02 Approximations

Subtopic: Approximations

Topic: Introduction

8. An ideal dc voltage source produces a load voltage

A. that is variable.

B. that is constant.

C. that is zero.

D. that is greater than 250 volts.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.03 Voltage Sources

Subtopic: Voltage Sources

Topic: Introduction

9. An ideal voltage source is a theoretical device because a real voltage source has

A.

A. an infinite load resistance.

B.

B. no internal resistance.

C.

C. an internal resistance.

D. no load current.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.03 Voltage Sources

Subtopic: Voltage Sources

Topic: Introduction

10. The term stiff voltage source refers to a voltage source with a load resistance which is

A.

A.  times larger than the source resistance.

B. 100 times larger than the source resistance.

C. equal to the source resistance.

D. 100 times smaller than the source resistance.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.03 Voltage Sources

Subtopic: Voltage Sources

Topic: Introduction

11. A dc current source produces a constant load current

A. for different load resistances.

B. for constant load resistances only.

C. by using a constant voltage source.

D. by using alternating current.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.04 Current Sources

Subtopic: Current Sources

Topic: Introduction

12. In the second approximation of a current source

A.

A. the internal resistance is in parallel with the ideal current source.

B.

B. the internal resistance is in series with the ideal current source.

C.

C. the internal resistance is zero.

D. the internal resistance is infinity.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.04 Current Sources

Subtopic: Current Sources

Topic: Introduction

13. Thevenin's theorem is a statement that can be proven

A.

A. mathematically.

B. by experiment only.

C. by using Ohm's Law.

D. by using Kirchoff's Laws.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

14. The Thevenin voltage is sometimes called

A. the open-circuit voltage.

B. the short-circuit voltage.

C. the source voltage.

D. the load voltage.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

15. What is a circuit often built with solderless connections without regard to the final location of parts?

A.

A. breadboard

B.

B. prototype

C.

C. circuit board

D. batter board

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

16. In order to avoid loading error, a field-effect transistor (FET) input or digital multimeter (DMM) is generally used to measure a voltage.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

17. Norton current is sometimes called the open-circuit current.

FALSE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.06 Norton’s Theorem

Subtopic: Norton’s Theorem

Topic: Introduction

18. Norton's theorem states that the load voltage equals the Norton current times the Norton resistance in parallel with the load resistance.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.06 Norton’s Theorem

Subtopic: Norton’s Theorem

Topic: Introduction

19. The duality principle states that for any theorem in electrical circuit analysis there is a dual (opposite) theorem in which one replaces the original quantities with dual quantities.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.06 Norton’s Theorem

Subtopic: Norton’s Theorem

Topic: Introduction

20. A Thevenin circuit cannot be converted to a Norton circuit.

FALSE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.06 Norton’s Theorem

Subtopic: Norton’s Theorem

Topic: Introduction

21. Troubleshooting means finding out why a circuit is not doing what it is supposed to do.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction

22. The most common circuit troubles are

A.

A. faulty power supplies.

B. improper design.

C. broken switches.

D. open and shorts.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction

23. A solder bridge

A. is a poor solder connection.

B. is also called a cold-solder joint.

C. effectively shorts any device between two conducting lines.

D. effectively opens any device between two conducting lines.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction

24. The current through an open device is

A.

A. zero.

B.

B. very high.

C.

C. always greater than 10 amperes.

D. measured with a voltmeter.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction

25. The voltage across a shorted device is

A.

A. zero.

B. very high.

C. always greater than 10 volts.

D. measured with an ammeter.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction

26. In a circuit, voltages are usually measured with respect to

A. the positive side of the power source.

B. the component known as the fuse.

C. ground.

D. the load.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction

27. In the troubleshooting procedure, after a component has been isolated as the top suspect,

A.

A. replace it right away.

B.

B. unsolder or disconnect it and use test equipment for confirmation.

C.

C. replace component with a two-inch piece of number 16 gauge wire.

D. test component in the circuit using an ohmmeter.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction

28. Definitions are formulas that a researcher creates.

FALSE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.01 The Three Kinds of Formulas

Subtopic: The Three Kinds of Formulas

Topic: Introduction

29. Coulomb's Law states that the force of attraction or repulsion between two charges is directly proportional to the charges and inversely proportional to the square of the distance between them.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.01 The Three Kinds of Formulas

Subtopic: The Three Kinds of Formulas

Topic: Introduction

30. A derivation is a formula that we can get from other formulas.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.01 The Three Kinds of Formulas

Subtopic: The Three Kinds of Formulas

Topic: Introduction

31. The different levels of approximation are an ideal approximation, a second approximation, a third approximation, and an exact answer.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.02 Approximations

Subtopic: Approximations

Topic: Introduction

32. When troubleshooting, the ideal approximation is never adequate.

FALSE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.02 Approximations

Subtopic: Approximations

Topic: Introduction

33. For higher approximations, a computer and a program like MultiSim should be used.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.02 Approximations

Subtopic: Approximations

Topic: Introduction

34. With an ideal voltage source, only the load current changes when the load resistance changes.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.03 Voltage Sources

Subtopic: Voltage Sources

Topic: Introduction

35. The second approximation of a dc voltage source includes internal resistance.

TRUE

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.03 Voltage Sources

Subtopic: Voltage Sources

Topic: Introduction

36. When can the source resistance be ignored?

A. when it is at least 100 times smaller than the load resistance

B. when it is at least 10 times smaller than the load resistance

C. when it is equal to the load resistance

D. never

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.03 Voltage Sources

Subtopic: Voltage Sources

Topic: Introduction

37. Using the second approximation for an ac voltage source is valid only at what frequencies?

A.

A. low

B.;B.

B. high

C.

C. ultrahigh

D. VHF

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.03 Voltage Sources

Subtopic: Voltage Sources

Topic: Introduction

38. A dc voltage source produces a constant load voltage

A.

A. for different load resistances.

B. for equal load resistances only.

C. with zero load resistance.

D. by using alternating current.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.03 Voltage Sources

Subtopic: Voltage Sources

Topic: Introduction

39. When can the source resistance of a current source be ignored?

A. when it is at least 100 times larger than the load resistance

B. when it is at least 10 times larger than the load resistance

C. when it is equal to the load resistance

D. when it is zero

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.04 Current Sources

Subtopic: Current Sources

Topic: Introduction

- A current source that has a source resistance that is at least 100 times larger than the load resistance is called a

A. stiff current source.

B.

B. staff current source.

C.

C. potential current source.

D. kinetic current source.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.04 Current Sources

Subtopic: Current Sources

Topic: Introduction

41. When analyzing a transistor circuit, the transistor is visualized as a

A.

A. load resistor.

B. voltage source.

C. current source.

D. voltmeter.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.04 Current Sources

Subtopic: Current Sources

Topic: Introduction

42. Thevenin voltage is defined as the voltage across the load terminals when

A. the voltage source is removed.

B. the load resistor is open.

C. the load resistor is shorted.

D. the voltage source is shorted.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

- Thevenin resistance is defined as the resistance that an ohmmeter measures across the load terminals when all sources are reduced to zero and

A.

A. the load resistor is open.

B. B.

B. the load resistor is shorted.

C. C.

C. the load resistor is replaced with a transistor.

D. the load resistor is replaced with a fuse.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

44. When you reduce a current source to zero

A.

A. you are effectively replacing it by a short.

B. you are effectively replacing it by an open.

C. you are effectively replacing it by a low-value resistor.

D. you are effectively replacing it by a fuse.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

45. A linear resistance

A. increases with increasing voltage.

B. decreases with increasing voltage.

C. does not change with increasing voltage.

D. is effectively nonlinear in a Thevenin circuit.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

46. Thevenin theorem simplifies calculations and

A.

A. enables us to explain circuit operation.

B.

B. troubleshoots transistor circuits.

C.

C. serves as a constant current source.

D. serves as a constant voltage source.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

47. Whenever two vertical bars are used in an equation, it means

A.

A. divide twice.

B. in series with.

C. in parallel with.

D. multiply the following value.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

48. A breadboard is

A. a shelf used to store components.

B. a circuit often built with solderless connections.

C. a small printed circuit board.

D. a schematic diagram of a circuit.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.05 Thevenin’s Theorem

Subtopic: Thevenin’s Theorem

Topic: Introduction

49. When measuring a voltage, what causes a small current to flow through the multimeter?

A.

A. a small load resistance.

B.

B. the D'Arsenval meter movement.

C.

C. the input impedance.

D. the output impedance.

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Easy

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction

50. The input impedance of a voltmeter should be how many times greater than the Thevenin resistance?

A. 10

B. 100

C. 1000

D. 2

Accessibility: Keyboard Navigation

Bloom's: 2. Understand

Chapter: 01 Introduction

Difficulty: Medium

Section: 01.07 Troubleshooting

Subtopic: Troubleshooting

Topic: Introduction